Flying supersonically on Concorde

The development of the aircraft Concorde was a joint venture between the French and the English and it first took to the skies in 1969, although it was not until 1976 that it flew commercially with passengers. Concorde then continued to fly for 27 years with the majority of flights being between Paris and New York for the French Concordes and between London and New York for the British Concordes. Flying these routes took Concorde half the time of traditional airliners.

In total there were 20 Concordes built but they did not all enter service flying passengers as six were a combination of prototypes and pre-production test models. Later in their life some of the Concordes were used for spare parts in order to keep the others flying.

Concorde was instantly recognizable with its double delta wings and was the only supersonic airliner on which passengers were able to travel at twice the speed of sound. Russia did develop a supersonic airliner but safety problems with the prototype airliners meant that it did not fly commercially.

One of the instantly recognizable features of Concorde is the nose section, which tilts at an angle when taking off or landing to enable the pilot to see the runway. The angle of tilt for take-off is less than that for landing because during landing the aircraft is pitched at a much larger angle.

Breaking the sound barrier is only done over seas to avoid any disturbance on the ground. For this reason, the flight plan may call for a levelling off at the subsonic speed of Mach 0.93 and at an altitude of 29 500 ft over the land after takeoff until water can be reached. Once Concorde is over water, it proceeds to climb rapidly to reach its cruising speed and altitude.

Since there is a change in the aerodynamic centre of the aircraft when going from subsonic to supersonic speed, fuel is pumped to the rear of the aircraft to compensate for this shift.

As Concorde accelerates, the resistance due to the surrounding air increases and the thrust from the engines needs to be increased considerably. You then feel two slight impulses a second apart as Concorde breaks the sound barrier. The machmeter in the cabin now reads Mach 1. After this the aircraft rapidly reaches Mach 2 (1350 mph) which is the aircraft's cruising speed.

Concorde's cruising altitude ranges between 52 000 feet to 59 000 feet which is around 11 miles high. Because this is too high for normal aircraft, the ideal cruising altitude is chosen by the captain rather than by air traffic control. At this height you are travelling in the stratosphere where the atmospheric pressure is only one tenth of that at the surface of the earth. Here the sky is an intense blue almost purple. Flying is very smooth as no weather disturbances occur at this height.

The range of Concorde carrying 100 passengers and nine crew under ideal conditions was 4500 miles, which enabled it to operate charter flights from London to Barbados in the winter months.

Comparison of flying times and speeds from Paris to New York

| Year | Aircraft | Flying time | Speed (km/h) |
| --- | --- | --- | --- |
| 1927 | Lindbergh | 33h 30min | 188 |
| 1946 | DC4 | 23h 45min | 350 |
| 1957 | Superstaliner | 14h 40min | 530 |
| 1959 | Boeing 707 | 8h | 900 |
| 1977 | Concorde | 3h 45min | 2200 |

The reason why there are no flying Concordes today is due to the cost of maintaining the aircraft. There were many safety modifications needed and the costs could not be recouped.

There was an unfortunate accident involving an Air France Concorde during take-off. A previous take-off of a DC10 aircraft cause tyre debris to be left on the runway and this flew up when Concorde ran over it causing the fuel tank to spill fuel. The fuel ignited and power was lost on one of the engines at a vital time during ascent of the aircraft. The aircraft lost thrust and fell out of sky killing everyone onboard.